# How to Specify the HL-490L LED
Helideck Semi-Flush Perimeter Light

The fixture shall be model HL-490L-(LED color)-(voltage) as marked by Flight Light Inc.

Construction:

•	The fixture shall be 6" [15.24 cm] diameter, 5.25" [13.34 cm] bolt circle.

•	The fixture shall be omnidirectional.

•	The fixture top material shall be 304 stainless steel with nitric acid passivation.

•	The fixture bottom material shall be 6061-T6 aluminum, hard anodized per MIL-A-8625 Type III Class 1, 1 mil.

•	The fixture shall have a modular design comprised of 2 major parts: Bottom Casting and Top Casting. The Bottom and Top Casting are sealed by means of a Prism Clamp and O-ring.

•	Hardware shall be 316 stainless steel.

•	Fixture shall ground internally.

•	The fixture shall include 48" [122 cm] of UL SO Cable 16AWG (UL Version).

LED/Light Cavity:

•	LED lifetime expectancy: 50,000 hours.

•	Power consumption shall not exceed 7.2VA for AC lights, 4W for DC lights, 10.8VA for IR AC lights and 7.2W for IR DC lights.

•	Infrared output wavelength: 850nm

•	Infrared output usable beam width: 55° to 125° vertical, 360° horizontal

•	Visible output wavelength: 530nm

•	Visible output beam width: 30° vertical, 360° horizontal

Compliances:

•	IP67 Compliant

•	Buy American Act Compliant

•	FAA AC 150/5390-2D Heliport Design Guide

•	ICAO Annex 14, Volume II for TLOF

•	Optical performance exceeds FAA AC 150/5390-2D Appendix G Specifications

•	Transport Canada TP14371, AGA 7.16

Options (to be specified per each project):

•	Infrared (must be used in conjunction with visible LEDs)

•	Dimming (only available with the Flight Light controller)

•	UL Listed fixture

•	6" [15.24 cm] Deep Base Can for ground installation.